NORTH-WEST UNIVERSITY

FACULTY OF NATURAL AND AGRICULTURAL SCIENCES

LABORATORY REPORT

EXPERIMENT 2

BASIC ASSEMBLY PROGRAMMING USING THE PIC16F84

Student Name: TSHEPO MNGOMEZULU

Student Number: 42090474

Module: ELYM328

Date Conducted: 24/08/2026

Submission Date: 31/08/2026

# AIM

The aim of this experiment was to develop, assemble, and test simple assembly-language programs for the PIC16F84 microcontroller using the Microchip MPLAB Integrated Development Environment (MPLAB IDE). The experiment focused on configuring PORTB as an output port, generating specified LED patterns, performing a basic arithmetic calculation, and verifying the resulting PORTB values using the MPLAB SIM software simulator.

# INTRODUCTION

The PIC16F84 is an 8-bit microcontroller that can be programmed using assembly language to perform digital control and processing tasks. Assembly programming provides direct access to the microcontroller registers and input/output ports, making it suitable for demonstrating fundamental microcontroller operations.

For this experiment, the Microchip MPLAB IDE was used to create and manage separate projects for Problems 1(a), 1(b), and 2. The MPASM Assembler was used to translate the assembly source code into machine-readable form, while the MPLINK Object Linker and MPLAB Librarian (MPLIB) formed part of the configured language-tool environment. A PIC16F84 assembly template and the corresponding linker file were added to each project.

PORTB is an 8-bit bidirectional digital port. Before PORTB can be used to display an output, its direction must be configured through the TRISB register. A logic 0 in a TRISB bit configures the corresponding PORTB pin as an output. The STATUS register's RP0 bit is used to select the appropriate register bank when accessing TRISB.

The experiment consisted of two LED-output problems and one arithmetic problem. Problem 1(a) required alternate LEDs to be illuminated through PORTB. Problem 1(b) required the LED pattern ON-ON-OFF-OFF-ON-ON-OFF-ON. Problem 2 required the calculation 7 + 15 + 208 and the display of the resulting value on PORTB.

# PROCEDURE

## Installation and Initial MPLAB IDE Setup

The Microchip MPLAB IDE software was installed on the laboratory computer and launched after installation. The MPLAB environment was then prepared for PIC16F84 assembly programming.

The Project Wizard was opened to establish a new microcontroller project. The PIC16F84 was selected as the target device. The required language tools were then configured, namely the MPASM Assembler, MPLINK Object Linker, and MPLIB Librarian. The installed language tools were assigned to their appropriate locations before proceeding with project creation.

## Creation of Project 1A

A separate project was created for Problem 1(a), with the project name and project directory specified through the Project Wizard. The PIC16F84 assembly template was located within the MPASM Suite template resources and the file 16F84TMPO.asm was added to the project. The corresponding PIC16F84 linker file, 16F84.lkr, was also located and added to the project.

After the required files had been added, the project view was opened through the MPLAB interface. The source file 16F84TMPO.asm was selected, displaying the PIC16F84 assembly template. The program instructions required for Problem 1(a) were inserted into the main program section of the template.

## Building and Simulating Project 1A

The completed Project 1A was assembled using the Build All command. The project was successfully built without errors. The simulator/debugger settings were then configured to use MPLAB SIM.

FIGURE1: Sample code used to build problem 1a.

The processor was reset using the Processor Reset function. The program was executed with instruction-by-instruction using Debugger Step Into. During execution, the green execution arrow was used to identify the current instructions.

The Watch window was opened to monitor the relevant special function registers. Since PORTB was used as the output port, the PORTB SFR was monitored during execution. The PORTB value was examined in binary representation to verify the alternating output pattern.

## Creation and Simulation of Project 1B

A separate MPLAB project was created for Problem 1(b) using the same PIC16F84 device and language-tool configuration. The 16F84TMPO.asm assembly template and 16F84.lkr linker file were added to the project.

The template source file was edited by inserting the assembly instructions required to produce the specified LED pattern. The project was then built using Build All and was successfully assembled without errors.

Figure 2: Build Output for Problem 1b

MPLAB SIM was selected as the debugging tool, after which the processor was reset. The program was stepped through using Step Into, and the Watch window was used to monitor PORTB. The binary PORTB output was checked against the required ON-ON-OFF-OFF-ON-ON-OFF-ON pattern.

## Creation and Simulation of Project 2

A third, separate MPLAB project was created for Problem 2. The PIC16F84 was selected, and the same language tools, assembly template, and linker file were incorporated into the project.

The assembly program was edited to calculate 7 + 15 + 208. A temporary register named TEMP was used to hold intermediate and final arithmetic results. After the arithmetic operation was completed, the final value stored in TEMP was transferred to the W register and subsequently written to PORTB.

Figure3: Build Output for Problem 2a

The project was assembled using Build All and was successfully built without errors. MPLAB SIM was selected, the processor was reset, and the program was executed step-by-step. The Watch window was used to observe the relevant SFR values, including PORTB, and the final result was examined in binary form.

## Verification of Results

After completing the three programming tasks, the programs and simulated results were presented to the student instructor. Questions concerning the assembly instructions, register configuration, program operation, and obtained results were answered successfully. The practical work was consequently verified, and the corresponding assessment marks were awarded.

# RESULTS

The expected outputs were determined from the assembled program instructions and the values written to PORTB. The outputs below correspond to the steady-state values produced by the programs.

## Problem 1(a): Alternate LEDs

The program loaded the binary value 10101010 into PORTB.

PORTB output:

RB7  RB6  RB5  RB4  RB3  RB2  RB1  RB0
  1    0    1    0    1    0    1    0

Binary: 10101010

Hexadecimal: AA

Decimal: 170

Expected LED state: ON-OFF-ON-OFF-ON-OFF-ON-OFF

Figure 4: MPLAB SIM Watch window showing the PORTB output for Problem 1(a).

## Problem 1(b): Required LED Pattern

The program loaded the binary value 11001101 into PORTB.

PORTB output:

RB7  RB6  RB5  RB4  RB3  RB2  RB1  RB0
  1    1    0    0    1    1    0    1

Binary: 11001101

Hexadecimal: CD

Decimal: 205

Expected LED state: ON-ON-OFF-OFF-ON-ON-OFF-ON

Figure 5: MPLAB SIM Watch window showing the PORTB output for Problem 1(b).

## Problem 2: Arithmetic Operation

The arithmetic operation performed by the program was:

7 + 15 + 208 = 230

The decimal result of 230 is equivalent to E6 in hexadecimal and 11100110 in binary.

PORTB output:

Binary:      11100110
Hexadecimal: E6
Decimal:     230

Figure 6: MPLAB SIM Watch window showing the calculated result on PORTB.

# DISCUSSION

### Question 1: Implementation of Problem 1(b)

For Problem 1(b), we were given a sample PIC16F84 assembly program/template file, 16F84TMPO.asm, which already contained the basic structure of the program. We created a separate MPLAB project for Problem 1(b), selected the PIC16F84 as the target device, and added the provided assembly template and the 16F84.lkr linker file to the project.

We then entered our own program instructions under the Main section of the provided template. First, we selected Bank 1 using BSF STATUS,RP0 so that the TRISB register could be accessed. We used CLRF TRISB to configure all the PORTB pins as outputs, and then used BCF STATUS,RP0 to return to Bank 0.

After configuring PORTB, we used MOVLW b'11001101' to load the required LED pattern into the W register. The instruction MOVWF PORTB then transferred the pattern from the W register to PORTB. The binary value 11001101 corresponds to the required LED pattern ON-ON-OFF-OFF-ON-ON-OFF-ON when read from RB7 to RB0.

The program was then built using Build All and successfully assembled without errors. MPLAB SIM was selected, the processor was reset, and the program was executed step-by-step using Step Into. The PORTB register was monitored in the Watch window to confirm that the required binary pattern was produced.

### Question 2: Explain how your program for Problem 2 works. with the purpose of each line of the code.

The purpose of the program was to calculate 7 + 15 + 208 and display the final result on PORTB. A temporary register called TEMP was used to store the intermediate and final results of the calculation. The final result obtained was 230, which is 11100110 in binary and E6 in hexadecimal.

The program begins with LIST P=16F84. This specifies that the program is written for the PIC16F84 microcontroller.

#INCLUDE <P16F84.INC> includes the PIC16F84 definitions, such as the names of the registers and bits used in the program.

__CONFIG _CP_OFF & _WDT_ON & _PWRTE_ON & _RC_OSC specifies the configuration settings for the microcontroller.

w_temp EQU 0x0C assigns memory address 0x0C to the temporary register used for storing the W register during interrupt handling.

status_temp EQU 0x0D assigns memory address 0x0D to the temporary register used for storing the STATUS register during interrupt handling.

TEMP EQU 0x0E assigns memory address 0x0E to the temporary register used to store the arithmetic results.

RESET_VECTOR CODE 0x000 defines the reset vector at program memory address 000.

GOTO Main causes the program to jump to the Main section when the microcontroller is reset.

ISR CODE 0x004 defines the interrupt service routine at program memory address 004.

GOTO Interrupt causes execution to jump to the interrupt routine when an interrupt occurs.

MAIN_PROGRAM CODE indicates the beginning of the main program section.

The Interrupt section contains the standard interrupt-handling code provided in the assembly template. MOVWF w_temp saves the current value of the W register in w_temp. MOVF STATUS,W copies the STATUS register into W, and MOVWF status_temp saves that value in status_temp. The following instructions restore the STATUS and W registers before RETFIE returns from the interrupt. These instructions were part of the supplied PIC16F84 assembly template and were not required for the arithmetic calculation itself.

The main program then starts at the Main label.

BSF STATUS,RP0 sets the RP0 bit in the STATUS register and selects Bank 1. Bank 1 is required to access the TRISB register.

CLRF TRISB clears all the bits in TRISB. Since a zero in a TRISB bit configures the corresponding PORTB pin as an output, this configures all eight PORTB pins as outputs.

BCF STATUS,RP0 clears the RP0 bit and returns the microcontroller to Bank 0, where PORTB can be accessed.

MOVLW .7 loads the decimal value 7 into the W register.

MOVWF TEMP transfers the value 7 from the W register into the temporary register TEMP. Therefore, TEMP now contains 7.

MOVLW .15 loads the decimal value 15 into the W register.

ADDWF TEMP,F adds the value in W, which is 15, to the value stored in TEMP, which is 7. The result is 22. The F indicates that the result is stored back in the file register, which in this case is TEMP.

MOVLW .208 loads the decimal value 208 into the W register.

ADDWF TEMP,F adds 208 to the current value in TEMP, which is 22. The result is 230, and the result is stored back in TEMP.

Therefore, the calculation performed by the program is:

7 + 15 = 22

22 + 208 = 230

MOVF TEMP,W copies the final value of 230 from TEMP into the W register.

MOVWF PORTB transfers the value from W to PORTB. Therefore, the value 230 is displayed through the eight PORTB output pins. The result is 11100110 in binary and E6 in hexadecimal.

Loop: is a label that identifies the beginning of an infinite loop.

GOTO Loop continuously sends the program back to the Loop label. This keeps the program running and maintains the final value on PORTB.

END indicates the end of the assembly source program.

Overall, the program first configures PORTB as an output, then stores 7 in TEMP, adds 15 to it, and finally adds 208. The resulting value of 230 is transferred to PORTB, where it can be observed in the MPLAB SIM Watch window.

The three programs demonstrated fundamental assembly programming operations on the PIC16F84. In each program, PORTB was configured as an output before data was written to the port. This configuration was achieved by selecting Bank 1 through the RP0 bit of the STATUS register, clearing TRISB, and then returning to Bank 0.

The instruction BSF STATUS,RP0 sets the RP0 bit and selects Bank 1. The instruction CLRF TRISB clears all eight bits of the TRISB register. Because cleared TRISB bits correspond to output configuration, all PORTB pins were configured as outputs. The instruction BCF STATUS,RP0 then clears RP0 and returns the program to Bank 0, where PORTB can be accessed.

In Problem 1(a), MOVLW b'10101010' loaded the alternating bit pattern into the W register. MOVWF PORTB then transferred this value to PORTB. Consequently, the eight output bits alternated between logic high and logic low, producing the required alternate LED illumination.

## 3(a) Bank Memory

The PIC16F84 uses banked register memory. Different special function registers are located in different memory banks. The STATUS register contains the RP0 bit used to select between the relevant register banks. In the programs used for this experiment, Bank 1 was selected to access TRISB, after which Bank 0 was selected to access PORTB.

## (b) Purpose of TRISB

TRISB determines the direction of the PORTB pins. A TRISB bit set to 1 configures the corresponding PORTB pin as an input, while a bit set to 0 configures it as an output. Clearing TRISB therefore configures all eight PORTB pins as outputs.

## (c) STATUS Register Bits

The STATUS register contains important processor-status information. In particular, RP0 is used for bank selection. The other STATUS bits include arithmetic and reset/status indicators, including the Carry (C), Digit Carry (DC), and Zero (Z) flags. These flags can be affected by arithmetic or logical operations and are useful when controlling program flow.

# CONCLUSION

The experiment successfully demonstrated basic assembly programming of the PIC16F84 microcontroller using MPLAB IDE and MPLAB SIM. Three separate projects were created, assembled, and simulated. Problem 1(a) generated the alternating PORTB pattern 10101010, while Problem 1(b) generated the required 11001101 LED pattern. Problem 2 successfully performed the arithmetic operation 7 + 15 + 208, producing 230 or 11100110 in binary. The experiment also provided practical experience with project creation, assembly, register-bank selection, TRISB configuration, PORTB output, step-by-step debugging, and verification using the MPLAB SIM Watch window.

# Appendix A: Assembly Programs

The following programs are the assembly source codes used for the three practical tasks.

## Appendix A.1: Assembly code for Problem 1(a)

LIST      P=16F84
#INCLUDE  <P16F84.INC>

__CONFIG  _CP_OFF & _WDT_ON & _PWRTE_ON & _RC_OSC

w_temp        EQU     0x0C
status_temp   EQU     0x0D

RESET_VECTOR    CODE    0x000
        GOTO    Main

ISR             CODE    0x004
        GOTO    Interrupt

MAIN_PROGRAM    CODE

Interrupt:
        MOVWF   w_temp
        MOVF    STATUS,W
        MOVWF   status_temp
        MOVF    status_temp,W
        MOVWF   STATUS
        SWAPF   w_temp,F
        SWAPF   w_temp,W
        RETFIE

Main:
        BSF     STATUS,RP0
        CLRF    TRISB
        BCF     STATUS,RP0

        MOVLW   b'10101010'
        MOVWF   PORTB

Loop:
        GOTO    Loop

        END

## Appendix A.2: Assembly code for Problem 1(b)

LIST      P=16F84
#INCLUDE  <P16F84.INC>

__CONFIG  _CP_OFF & _WDT_ON & _PWRTE_ON & _RC_OSC

w_temp        EQU     0x0C
status_temp   EQU     0x0D

RESET_VECTOR    CODE    0x000
        GOTO    Main

ISR             CODE    0x004
        GOTO    Interrupt

MAIN_PROGRAM    CODE

Interrupt:
        MOVWF   w_temp
        MOVF    STATUS,W
        MOVWF   status_temp
        MOVF    status_temp,W
        MOVWF   STATUS
        SWAPF   w_temp,F
        SWAPF   w_temp,W
        RETFIE

Main:
        BSF     STATUS,RP0
        CLRF    TRISB
        BCF     STATUS,RP0

        MOVLW   b'11001101'
        MOVWF   PORTB

Loop:
        GOTO    Loop

        END

## Appendix A.3: Assembly code for Problem 2

LIST      P=16F84
#INCLUDE  <P16F84.INC>

__CONFIG  _CP_OFF & _WDT_ON & _PWRTE_ON & _RC_OSC

w_temp        EQU     0x0C
status_temp   EQU     0x0D
TEMP          EQU     0x0E

RESET_VECTOR    CODE    0x000
        GOTO    Main

ISR             CODE    0x004
        GOTO    Interrupt

MAIN_PROGRAM    CODE

Interrupt:
        MOVWF   w_temp
        MOVF    STATUS,W
        MOVWF   status_temp
        MOVF    status_temp,W
        MOVWF   STATUS
        SWAPF   w_temp,F
        SWAPF   w_temp,W
        RETFIE

Main:
        BSF     STATUS,RP0
        CLRF    TRISB
        BCF     STATUS,RP0

        MOVLW   .7
        MOVWF   TEMP

        MOVLW   .15
        ADDWF   TEMP, F

        MOVLW   .208
        ADDWF   TEMP, F

        MOVF    TEMP, W
        MOVWF   PORTB

Loop:
        GOTO    Loop

        END

REFERENCES
MPLAB IDE

Quick Start Guide.

PIC16F84 DATA SHEET

PICmicro Mid-Range MCU Family Reference Manual.